SOLARPUNK PI

DATASHEET

RK3576 + RP2350 + RK3506J

Triple-Processor Solar-Powered Edge Computer

85×56mm Pi 5 Form Factor • 6-Layer PCB • 8GB LPDDR4X • 32GB eMMC • NVMe

WiFi 5 + BT 5.0 • GbE + PoE • 4G LTE + nano-SIM • LoRa Mesh • 6 TOPS NPU

2× MIPI CSI Camera • Micro-HDMI 4K • 3.5mm Stereo Audio • ES8316 Codec

2× USB-C 3.1 + 2× USB-C 2.0 • 40-Pin GPIO • 10-Pin RP2350 Expansion

30-Pin Industrial: CAN FD×2 • RS485 • PWM×12 • ADC×4 • UART×4 • SPI×2 • I2C×2

Solar MPPT + LiFePO4 • PoE 802.3at • USB-C PD • Auto-OR Switching

Schmitt Trigger Signal Conditioning • Ferrite Bead Power Filtering • ESD Protection

Docker • Ollama • Reticulum • FreeRTOS • AMP Multi-Core

| RK3576 — Brain 8-core + 6T NPU Linux, Docker, Ollama | RP2350 — Nerves 2×M33, 0.1W Power, LoRa, WiFi | RK3506J — Muscle 3×A7+M0, 0.7W CAN, RS485, motors |
| --- | --- | --- |

Rev 3.0 • March 2026 • Preliminary

Table of Contents

# 1. General Description

Solarpunk Pi is a triple-processor single-board computer designed for solar-powered edge computing, AI inference, mesh networking, and industrial automation. It integrates three Rockchip/Raspberry Pi processors on an 85×56mm Pi 5-compatible PCB, each serving a distinct role: high-performance Linux computing (RK3576), ultra-low-power radio and power management (RP2350), and hard real-time industrial control (RK3506J).

The board accepts power from solar panels, PoE Ethernet, or USB-C PD with automatic source switching. All inter-chip communication lines are Schmitt-trigger buffered for noise immunity, and all power domains are ferrite-bead filtered for stability. Industrial I/O channels are galvanically isolated for safe connection to factory equipment.

## 1.1 Key Features

- Triple-processor architecture: RK3576 (Linux) + RP2350 (power/radio) + RK3506J (RTOS automation)

- 6 TOPS NPU for on-device AI inference (Ollama, RKNN, TensorFlow Lite)

- 8GB LPDDR4X RAM + 32GB eMMC + M.2 NVMe SSD slot

- Five network interfaces: GbE, WiFi 5, 4G LTE, LoRa mesh, backup WiFi 4

- Dual MIPI CSI camera ports (4K + 1080p) + Micro-HDMI 4K + stereo audio

- 4× USB-C ports (2× USB 3.1 + 2× USB 2.0, all native, no hub)

- 30-pin industrial header with CAN FD×2, RS485, PWM×12, ADC×4, isolated

- Triple power input: Solar MPPT + PoE 802.3at + USB-C PD, auto-switching

- Schmitt trigger signal conditioning on all inter-chip and industrial signals

- 6-layer impedance-controlled PCB, ENIG finish, JLCPCB-optimized

# 2. System Architecture

Figure 1-1: System architecture with all three domains and Schmitt-triggered links

## 2.1 Processing Domains

| Domain | Processor | Always On | Key Peripherals | Power |
| --- | --- | --- | --- | --- |
| RK3576 | 4×A72@2.2G+4×A53@1.8G, 6T NPU | No (switched) | NVMe, eMMC, GbE, WiFi5, cellular, CSI×2, HDMI, audio, USB×4 | 3–8W |
| RP2350 | 2×Cortex-M33@150MHz, 520KB SRAM | Yes | CYW43439 WiFi/BT, SX1262 LoRa, ADC, power MOSFET gates | ~0.1W |
| RK3506J | 3×Cortex-A7@1.5G+Cortex-M0@200M | Yes | CAN×2, UART×6, SPI×3, I2C×3, PWM×12, ADC×4, RMII×2, USB×2 | ~0.7W |

## 2.2 Inter-Chip Communication

All signals crossing power domain boundaries pass through Schmitt trigger buffers (SN74LVC2G17 or SN74LVC1G17) for clean signal transitions, noise rejection, and ESD protection.

| Signal | From | To | Type | Speed | Schmitt Buffer |
| --- | --- | --- | --- | --- | --- |
| UART1 TX/RX | RK3576 UART4 | RP2350 GP0/GP1 | UART | 115200 | SN74LVC2G17 (dual) |
| UART2 TX/RX | RK3576 UART5 | RK3506J UART2 | UART | 921600 | SN74LVC2G17 (dual) |
| SPI bridge | RK3576 SPI2 | RK3506J SPI0 | SPI | 10MHz | Direct (same 3.3V) |
| Power enable | RP2350 GP15 | PMOS gate | GPIO | — | SN74LVC1G17 (single) |
| Power good | RK806 PWRGD | RP2350 GP12 | GPIO | — | SN74LVC1G17 (single) |
| RK3506 reset | RP2350 GP14 | RK3506J RESET | GPIO | — | Direct |
| Wake request | RK3506J GPIO | RP2350 GP13 | GPIO | — | SN74LVC1G17 (single) |
| Alarm IRQ | RK3506J GPIO | RK3576 GPIO | GPIO | — | SN74LVC1G17 (single) |
| Shutdown | RK3576 GPIO | RP2350 GP28 | GPIO | — | SN74LVC1G17 (single) |

# 3. Electrical Specifications

## 3.1 Absolute Maximum Ratings

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Solar input voltage | –0.3 | 22 | V |
| PoE input voltage (at RJ45) | 36 | 57 | V |
| USB-C PD input voltage | 4.5 | 20 | V |
| LiFePO4 battery voltage (2S) | 4.0 | 7.4 | V |
| RK3576 domain supply (5V_SYS) | 4.75 | 5.25 | V |
| I/O voltage (RK3576 / RK3506J) | 0 | 3.63 | V |
| I/O voltage (RP2350) | 0 | 3.63 | V |
| Operating temperature | –20 | +70 | °C |
| Storage temperature | –40 | +85 | °C |
| ESD tolerance (HBM) | — | 2000 | V |

## 3.2 Recommended Operating Conditions

| Parameter | Min/Typ/Max | Unit | Notes |
| --- | --- | --- | --- |
| Solar panel voltage | 6 / 12–18 / 20 | V | CN3722 MPPT range |
| Solar panel power | 10 / 50–100 / 120 | W | 100W recommended for 24/7 |
| LiFePO4 capacity | 5 / 20 / 50 | Ah | 20Ah recommended |
| PoE class | Class 4 (802.3at) | 25.5W | At PD, after cable losses |
| USB-C PD voltage | 12 | V | HUSB238 negotiates 12V |
| USB-C PD current | — / 2 / 3 | A | 24W typical available |
| Ambient temperature | 0 / 25 / 50 | °C | No heatsink needed <40°C |

## 3.3 Power Consumption

| System State | Typical | Maximum | Notes |
| --- | --- | --- | --- |
| Deep sleep (RP2350 only) | 0.2W | 0.3W | LoRa listening, ADC polling |
| Automation idle (RP2350+RK3506J) | 0.8W | 1.2W | CAN/RS485 active, no motor load |
| Linux idle (all three, no load) | 4W | 5W | Linux booted, Docker idle |
| Full compute (Ollama inference) | 10W | 12W | NPU + CPU active, WiFi+GbE |
| Peak load (all peripherals) | 14W | 18W | Cameras+cellular+NVMe+compute |

# 4. Connectivity & Pin Descriptions

## 4.1 External Connectors

| Connector | Type | Interface | Specification |
| --- | --- | --- | --- |
| USB-C #1 | USB-C female, top edge | USB 3.1 Gen1 OTG + DP Alt | 5Gbps data + 4K display + PD power in (12V) |
| USB-C #2 | USB-C female, top edge | USB 3.1 Gen1 Host | 5Gbps. Combo PHY (muxed w/ SATA). |
| USB-C #3 | USB-C female, right edge | USB 2.0 Host | 480Mbps. General peripherals. |
| USB-C #4 | USB-C female, right edge | USB 2.0 Host | 480Mbps. General peripherals. |
| RJ45 | 8P8C w/ PoE magnetics | GbE + PoE 802.3at | 1Gbps Ethernet + 25.5W PoE input |
| Micro-HDMI | Type-D female, left edge | HDMI 2.0 TX | 4K@60fps output |
| 3.5mm TRRS | Audio jack, left edge | Stereo out + mic in | ES8316 I2S codec, line-level |
| CSI #1 | 22-pin FPC, left edge | 4-lane MIPI CSI (dcphy) | Up to 4K@30fps, 48MP stills |
| CSI #2 | 15-pin FPC, left edge | 2-lane MIPI CSI (dphy) | Up to 1080p@60fps |
| Nano-SIM | Push-push, right edge | SIM card | For Quectel EC25 4G LTE cellular |
| Solar input | JST VH 2-pin, left edge | 6–20V DC | To CN3722 MPPT charger |
| Battery | JST PH 2-pin, left edge | 6.4V nominal | 2S LiFePO4 connection |
| U.FL ×3 | SMD, board edges | RF antenna | LoRa (1), WiFi main/aux (2) |

## 4.2 40-Pin GPIO Header (Pi 5 Compatible)

Standard Raspberry Pi 5 pinout. Active RK3576 GPIO bank. Directly compatible with Pi 5 HATs. I2C, SPI, UART, PWM, and general-purpose GPIO available per Pi 5 specification.

## 4.3 10-Pin RP2350 Expansion Header

| Pin | GPIO | Functions | Future Use |
| --- | --- | --- | --- |
| 1 | GP23 | GPIO / PWM / PIO | NeoPixel, IR, 1-Wire |
| 2 | GP24 | GPIO / PWM / PIO | Encoder, button matrix |
| 3 | GP25 | GPIO / PWM | Status LED, buzzer |
| 4 | GP29 | ADC3 / GPIO | Analog sensor (0–3.3V) |
| 5 | GP9 | GPIO / UART1 RX | Spare serial, interrupt |
| 6 | SWCLK | SWD debug | Programming probe |
| 7 | SWDIO | SWD debug | Programming probe |
| 8 | 3V3_RP | Power | 100mA always-on output |
| 9 | 5V_SYS | Power | 200mA fused output |
| 10 | GND | Ground | Reference |

## 4.4 30-Pin RK3506J Industrial Header

| Pin | Signal | Interface | Notes |
| --- | --- | --- | --- |
| 1–2 | CAN1_H / CAN1_L | CAN FD | Isolated (ADUM1401). 120Ω term jumper. 5Mbps. |
| 3–4 | CAN2_H / CAN2_L | CAN FD | Independent isolated bus. 120Ω term jumper. |
| 5–6 | RS485_A / RS485_B | RS485 | Isolated (ADUM1401). Modbus RTU. 120Ω term. |
| 7–8 | UART3_TX / UART3_RX | UART 3.3V | Spare serial. Schmitt buffered input. |
| 9–10 | UART4_TX / UART4_RX | UART 3.3V | 2nd spare serial. Schmitt buffered input. |
| 11–14 | PWM_CH0–CH3 | PWM out | 4 of 12 PWM channels. Motors, servos, LEDs. |
| 15–18 | ADC_IN0–ADC_IN3 | 12-bit ADC | 4-ch SAR ADC. 0–3.3V. RC filtered inputs. |
| 19–22 | GPIO_0–GPIO_3 | Digital I/O | 3.3V. Schmitt buffered. Limit switches, relays. |
| 23–24 | I2C1_SDA / I2C1_SCL | I2C bus 1 | Sensor bus (temp, pressure, humidity). |
| 25–26 | I2C2_SDA / I2C2_SCL | I2C bus 2 | IMU, magnetometer, barometer. |
| 27 | SPI2 (SCK/MOSI/MISO/CS) | SPI | External ADC/DAC, SPI sensor, OLED display. |
| 28 | +5V_OUT | Power | Fused 500mA for external sensors. |
| 29 | +3V3_OUT | Power | 200mA for low-power sensors. |
| 30 | GND | Ground | Star ground point. |

RK3506J Spare Peripherals (Not on Header) Beyond the 30-pin header, the RK3506J still has available: PWM×8 more channels, UART×2 more, USB 2.0 OTG ×2, RMII Ethernet ×2 (100Mbps), SPDIF TX+RX, PDM RX 8-ch, SAI (I2S) ×2, FLEXBUS/DSMC high-speed bus, GPIO ×20+, Watchdog ×2, Timer 12-ch. These can be accessed via a custom carrier board or by repurposing existing header pins in firmware.

# 5. Power System

Figure 2-1: Power flow with ferrite bead filtering and RC-filtered ADC monitoring

## 5.1 Power Sources

| Source | Input V | Conversion Path | Priority | Max Power |
| --- | --- | --- | --- | --- |
| Solar + LiFePO4 | 6–20V | CN3722 MPPT → 2S LiFePO4 → TPS61022 boost → 5V | 1 (highest) | 100W panel / 4A boost |
| PoE 802.3at | 48V | SI3402-B PD → isolated DC-DC → 5V | 2 | 25.5W at PD |
| USB-C PD | 12V | HUSB238 → buck converter → 5V | 3 | 24W (12V/2A) |
| LiFePO4 alone | 6.4V | TPS61022 boost → 5V | 4 (backup) | 20W (4A boost) |

## 5.2 Power Rails

| Rail | Voltage | Source | Powers | Always On |
| --- | --- | --- | --- | --- |
| SOLAR_IN | 6–20V | Panel via JST VH | CN3722 MPPT | N/A |
| VBAT | 6.4V | 2S LiFePO4 via JST PH | TPS61022 input | N/A |
| 48V_POE | 48V | PoE cable via RJ45 | SI3402-B PD | N/A |
| 5V_SYS | 5.0V | TPS61022 / PoE / USB-C (auto) | Entire board | Yes |
| 3V3_RP | 3.3V | MIC5219 LDO from 5V | RP2350, CYW, SX1262 | Yes |
| 3V3_RK3506 | 3.3V | AP2112K LDO from 5V | RK3506J, CAN, RS485 | Yes |
| VDD_CPU_BIG | 1.0V | RK806 DCDC1 | A72 cores | No |
| VDD_CPU_LIT | 0.9V | RK806 DCDC2 | A53 cores | No |
| VDD_GPU | 0.9V | RK806 DCDC3 | Mali-G52 | No |
| VDD_NPU | 0.9V | RK806 DCDC4 | 6T NPU | No |
| VDD_LOGIC | 0.75V | RK806 DCDC5 | RK3576 logic | No |
| VCC_DDR | 1.1V | RK806 DCDC6 | LPDDR4X ×2 | No |
| 3V3_RK | 3.3V | RK806 LDO | GbE, WiFi, eMMC, cell, audio | No |
| 1V8 | 1.8V | RK806 LDO | SDIO, eMMC, CSI, HDMI I/O | No |

## 5.3 Power Filtering & Stability

| Location | Component | Purpose |
| --- | --- | --- |
| 5V entry to each domain (3×) | Ferrite bead BLM18AG102SN1D | Block HF switching noise between domains |
| RP2350 ADC inputs (GP26/27) | 100Ω + 100nF RC filter per ch | Low-pass filter on VBAT/solar voltage dividers |
| RK3506J ADC inputs (4-ch) | 100Ω + 100nF RC filter per ch | Low-pass filter on industrial analog inputs |
| Every RK3576 VDD ball | 100nF 0402 within 0.5mm | Mandatory per-pin decoupling |
| Every domain entry point | 10µF 0805 ceramic | Bulk energy storage per domain |
| RK806 outputs | Per datasheet: bulk + MLCC | PMIC output capacitor requirements |

# 6. Signal Conditioning & Protection

## 6.1 Schmitt Trigger Buffers

All inter-chip communication signals and industrial digital inputs pass through Schmitt trigger buffers. This provides noise immunity (hysteresis), clean signal transitions across power domain boundaries, and basic ESD protection.

| Location | IC | Pkg | Qty | Signals Covered |
| --- | --- | --- | --- | --- |
| UART1 (RK3576↔RP2350) | SN74LVC2G17 | SOT-23-6 | 1 | TX + RX (2 channels) |
| UART2 (RK3576↔RK3506J) | SN74LVC2G17 | SOT-23-6 | 1 | TX + RX (2 channels) |
| GPIO power-good | SN74LVC1G17 | SOT-23-5 | 1 | RK806 PWRGD → RP2350 |
| GPIO wake request | SN74LVC1G17 | SOT-23-5 | 1 | RK3506J → RP2350 |
| GPIO alarm IRQ | SN74LVC1G17 | SOT-23-5 | 1 | RK3506J → RK3576 |
| GPIO shutdown | SN74LVC1G17 | SOT-23-5 | 1 | RK3576 → RP2350 |
| GPIO power gate | SN74LVC1G17 | SOT-23-5 | 1 | RP2350 → PMOS |
| Industrial header inputs (6ch) | SN74LVC14A | TSSOP-14 | 1 | GPIO×4 + UART3/4 RX inputs |
| Total |  |  | 8 ICs | 13 conditioned channels |

## 6.2 ESD & Overvoltage Protection

| Location | IC | Notes |
| --- | --- | --- |
| USB-C ports (×4) | USBLC6-2SC6 (×4) | Bidirectional TVS for USB D+/D− lines |
| Micro-HDMI | PRTR5V0U2X | HDMI-specific ESD clamp, low capacitance |
| Nano-SIM | PRTR5V0U2X | ESD on SIM data/clock lines |
| CAN / RS485 | ADUM1401 isolation | 2.5kV galvanic isolation IS the protection |
| Industrial GPIO inputs | PESD5V0S4UG | Quad TVS for unprotected digital inputs |
| RF antenna U.FL | Low-C TVS (<0.5pF) | On LoRa/WiFi paths, minimal capacitive loading |

# 7. PCB Specification

## 7.1 Board Parameters

| Parameter | Value |
| --- | --- |
| Board size | 85.0 × 56.0 mm (Pi 5 form factor) |
| Layer count | 6 layers |
| Stackup | JLCPCB JLC06161H-3313 |
| Surface finish | ENIG (Electroless Nickel Immersion Gold) |
| Min trace/space outer | 0.1mm / 0.1mm |
| Min trace/space inner | 0.09mm / 0.09mm |
| Via size | 0.4mm pad / 0.2mm drill (standard) |
| Impedance control | Required — specify stackup in order notes |
| Copper weight | 1 oz outer, 0.5 oz inner |
| Thickness | 1.6mm |
| Mounting holes | 4× 2.7mm at Pi 5 positions |
| Assembly | JLCPCB PCBA, both sides, X-ray for BGAs |

## 7.2 Layer Stackup

| Layer | Function | Impedance | Critical Signals |
| --- | --- | --- | --- |
| L1 — Top | RK3576 BGA fanout, DDR CH0 | 50Ω SE ref to L2 | DDR data, SDIO, USB, CSI, RGMII |
| L2 — GND | Solid ground pour | — | NEVER CUT. Reference for L1. |
| L3 — Signal | DDR CH1, NVMe, HDMI, USB3 | 85Ω diff | PCIe x2, HDMI TMDS, USB3 SS |
| L4 — Power | Copper fills: 5V, 3V3, 1V8, VBAT | — | Power planes, not traces |
| L5 — GND | Solid ground pour | — | NEVER CUT. Reference for L6. |
| L6 — Bottom | RP2350, RK3506J, LoRa, MPPT, PoE | 50Ω SE ref to L5 | SPI, LoRa RF, CAN/RS485 |

## 7.3 Impedance-Controlled Net Classes

| Net Class | Target | Trace W | Gap | Via | Use For |
| --- | --- | --- | --- | --- | --- |
| Default | 50Ω SE | 0.15mm | 0.15mm | 0.2mm | General routing |
| DDR_DQ | 50Ω SE | 0.10mm | 0.10mm | 0.2mm | DDR data lines |
| DDR_CLK | 85Ω diff | 0.09mm | 0.10mm | 0.2mm | DDR clock pairs |
| USB_HS | 90Ω diff | 0.085mm | 0.12mm | 0.2mm | USB 2.0 D+/D− |
| PCIE | 85Ω diff | 0.12mm | 0.12mm | 0.2mm | NVMe PCIe x2 |
| HDMI | 100Ω diff | 0.075mm | 0.15mm | 0.2mm | HDMI TMDS pairs |
| MIPI_CSI | 100Ω diff | 0.10mm | 0.12mm | 0.2mm | Camera MIPI lanes |
| SDIO | 50Ω SE | 0.15mm | 0.15mm | 0.2mm | WiFi 5 SDIO bus |
| I2S_AUDIO | 50Ω SE | 0.15mm | 0.15mm | 0.2mm | Audio codec I2S |
| POWER | — | 0.50mm | 0.25mm | 0.3mm | Power, battery traces |

# 8. Component Placement

Figure 3-1: RK3506J complete peripheral map

## 8.1 Placement Order

| # | Component | Side | Critical Rule |
| --- | --- | --- | --- |
| 1 | RK3576 BGA | TOP | ANCHOR — center-left. All others relative. |
| 2 | LPDDR4X ×2 | TOP | Above/below RK3576. Gap ≤3mm. DDR sandwich. |
| 3 | RK806 PMIC | TOP | Within 5mm right of RK3576. |
| 4 | eMMC + SPI NOR | TOP | Near FSPI pins. Short HS400 traces. |
| 5 | RTL8211F + RJ45 | TOP | Right edge. RGMII <40mm. |
| 6 | RTL8852BS WiFi | TOP | Top-right. 3mm antenna keepout ALL layers. |
| 7 | ES8316 + audio jack | TOP | Left edge. Near I2S pins. |
| 8 | Micro-HDMI + CSI ×2 | TOP | Left edge. TMDS/MIPI diff pairs short. |
| 9 | NVMe M.2 | TOP/BOT | Top edge or bottom side. PCIe diff. |
| 10 | Cellular M.2 + SIM | TOP | Near USB-C. USB 2.0. |
| 11 | USB-C ×4 | TOP | #1+#2 top edge, #3+#4 right edge. |
| 12 | 40-pin GPIO | TOP | Bottom edge. Pi 5 exact position. |
| 13 | RK3506J | BOTTOM | Bottom-left quadrant. Own 3V3 domain. |
| 14 | CAN/RS485 + ADUM1401 | BOTTOM | Near RK3506J. Isolation barrier. |
| 15 | SN74LVC14A Schmitt | BOTTOM | Between RK3506J and industrial header. |
| 16 | 30-pin industrial header | BOTTOM | Left edge. |
| 17 | RP2350A | BOTTOM | Bottom-right quadrant. Own 3V3 domain. |
| 18 | CYW43439 + SX1262 | BOTTOM | Near RP2350. 3mm RF keepout. |
| 19 | Inter-chip Schmitt ICs | BOTTOM | At domain boundaries. |
| 20 | CN3722 + TPS61022 | BOTTOM | Bottom-left. Away from RF/DDR. |
| 21 | PoE PD circuit | BOTTOM | Near RJ45 (via through-board). |
| 22 | 10-pin RP2350 header | BOTTOM | Right edge. |

# 9. Bus & Interface Allocation

Figure 4-1: RK3576 interface map and full RK3506J peripheral allocation

## 9.1 RK3576 Interface Map

| RK3576 Interface | Type | Connected To | Bandwidth / Notes |
| --- | --- | --- | --- |
| PCIe 2.1 x2 | Serial | NVMe M.2 2230 SSD | ~1 GB/s sequential |
| SDIO 3.0 | Parallel | RTL8852BS WiFi 5 + BT5 | 104MHz. Zero PCIe cost. |
| RGMII | Parallel | RTL8211F GbE PHY | 1 Gbps. Zero PCIe cost. |
| USB 3.1 OTG0 | Serial | USB-C #1 (+DP Alt+PD) | 5 Gbps + 4K display |
| USB 3.1 OTG1 | Serial | USB-C #2 (host) | 5 Gbps. Combo PHY (mux SATA). |
| USB 2.0 Host ×2 | Serial | USB-C #3 + #4 | 480 Mbps each |
| USB 2.0 Host (int) | Serial | Quectel EC25 cellular | Internal. 480 Mbps. |
| MIPI CSI (4L dcphy) | Serial | Camera #1 FPC | Up to 4K@30fps, 10 Gbps |
| MIPI CSI (2L dphy) | Serial | Camera #2 FPC | Up to 1080p@60fps, 5 Gbps |
| HDMI 2.0 TX | Serial | Micro-HDMI | 4K@60fps, 14.4 Gbps |
| I2S | Serial | ES8316 audio codec | Stereo audio |
| eMMC HS400 | Parallel | 32GB Samsung eMMC | 200 MB/s |
| FSPI | Serial | W25Q128 SPI NOR | Boot flash |
| UART4 | Serial | RP2350 (Schmitt) | 115200 baud |
| UART5 | Serial | RK3506J (Schmitt) | 921600 baud |
| SPI2 | Serial | RK3506J SPI0 | 10 MHz bulk |

## 9.2 RK3506J Full Peripheral Map

The RK3506J has significantly more peripherals than allocated on the 30-pin header. This table shows the complete set and allocation status:

| Peripheral | Total | On Header | Remaining / Notes |
| --- | --- | --- | --- |
| UART | 6 | 2 (UART3/4) | UART0=debug, UART2=RK3576 link. 2 spare internal. |
| SPI | 3 | 1 (SPI2) | SPI0=RK3576 bridge. 1 spare internal. |
| I2C | 3 | 2 (I2C1/2) | I2C0=internal. Both exposed. |
| CAN FD | 2 | 2 (both) | Both isolated. Fully exposed. |
| PWM | 12 | 4 (CH0–CH3) | 8 more available via GPIO mux or custom header. |
| SAR ADC | 4 ch | 4 (all) | All 4 channels exposed with RC filters. |
| USB 2.0 OTG | 2 | 0 | Available for USB sensors or serial adapters. |
| RMII Ethernet | 2 | 0 | 100Mbps each. Industrial Ethernet segments. |
| SPDIF | TX+RX | 0 | Digital audio. Ultrasonic sensor processing. |
| PDM | 8-ch RX | 0 | Digital microphone array. |
| SAI (I2S) | 2 ports | 0 | Audio serial interface. |
| FLEXBUS/DSMC | 1 | 0 | High-speed bus for FPGA or external ADC/DAC. |
| GPIO | ~32 | 4 (GPIO0–3) | ~28 more available via pin mux. |
| Watchdog | 2 | N/A | Hardware watchdog for fail-safe. |
| Timer | 12 ch | N/A | PID loops, encoder counting. |
| Cortex-M0 | 1@200MHz | N/A | Hard real-time ISR handler. |

# 10. Bill of Materials

## 10.1 RK3576 Compute Domain (17 components, top side)

| Component | Part Number | Package | Src | Notes |
| --- | --- | --- | --- | --- |
| RK3576 SoC | Rockchip RK3576 | FCCSP-698L | LCSC | X-ray required |
| LPDDR4X ×2 | Samsung K4UBE3D4AB | BGA-200 | Ali | 4GB each = 8GB |
| RK806 PMIC | RK806-1 | QFN-68 | LCSC | ArmSoM Sige5 rails |
| eMMC 32GB | Samsung KLMAG2JENB | BGA-153 | LCSC | HS400 |
| SPI NOR 16MB | W25Q128JVSIQ | SOP-8 | JLC | Bootloader |
| WiFi 5 + BT5 | RTL8852BS | LCC | Ali | SDIO 3.0 |
| GbE PHY | RTL8211F-CG | QFN-40 | LCSC | RGMII |
| Audio codec | ES8316 | QFN-24 | LCSC | I2S, stereo |
| Cellular | Quectel EC25-E | M.2 B-key | Ali | 4G LTE, USB 2.0 |
| NVMe slot | M.2 M-key 2230 | SMD | LCSC | PCIe x2 |
| CSI FPC ×2 | 22-pin + 15-pin | SMD | LCSC | MIPI cameras |
| Micro-HDMI | Type-D female | SMD | LCSC | 4K@60 |
| 3.5mm TRRS | Audio jack | THT | LCSC | Stereo + mic |
| USB-C ×4 | HRO TYPE-C-31-M-12 | SMD | JLC | #1:OTG+DP+PD |
| RJ45+magnetics | HR911105A w/PoE | THT | LCSC | GbE+PoE |
| Nano-SIM | Push-push | SMD | LCSC | Cellular SIM |
| 40-pin GPIO | 2×20 2.54mm | THT | LCSC | Pi 5 HAT |

## 10.2 RP2350 Domain (8 components, bottom side)

| Component | Part Number | Package | Src | Notes |
| --- | --- | --- | --- | --- |
| RP2350A MCU | RP2350A | QFN-60 | LCSC | Always-on |
| Flash 2MB | W25Q16JVSSIQ | SOP-8 | JLC | QSPI |
| WiFi/BT | CYW43439 | BGA | Ali | WiFi4+BT5.2 |
| Crystal | ABM8-12.000MHz | 3225 | JLC | Clock ref |
| LoRa | SX1262IMLTRT | QFN-24 | LCSC | 868/915MHz |
| U.FL antenna | U.FL-R-SMT-1(10) | SMD | LCSC | LoRa |
| 10-pin header | 1×10 2.54mm | THT | LCSC | Expansion |
| Modem MOSFET | Si2301 | SOT-23 | JLC | Cellular pwr |

## 10.3 RK3506J Domain (9 components, bottom side)

| Component | Part Number | Package | Src | Notes |
| --- | --- | --- | --- | --- |
| RK3506J SoC | Rockchip RK3506J | QFN | LCSC | 3×A7+M0 |
| DDR3L | 512MB LPDDR3L | BGA | LCSC | RTOS memory |
| NAND | 256MB NAND | BGA | LCSC | FW + logs |
| CAN FD ×2 | MCP2562FD ×2 | SOT-23-8 | JLC | Isolated |
| RS485 | SP3485 | SOP-8 | JLC | Modbus |
| Isolation | ADUM1401 | SOIC-16 | LCSC | 2.5kV galvanic |
| Schmitt (ind.) | SN74LVC14A | TSSOP-14 | JLC | 6-ch input clean |
| ESD (ind.) | PESD5V0S4UG | SOT-553 | LCSC | Quad TVS |
| 30-pin header | 3×10 2.54mm | THT | LCSC | Industrial I/O |

## 10.4 Signal Conditioning (8 ICs)

| Component | Part Number | Package | Qty | Purpose |
| --- | --- | --- | --- | --- |
| Dual Schmitt buffer | SN74LVC2G17 | SOT-23-6 | 4 | UART + GPIO boundaries |
| Single Schmitt buffer | SN74LVC1G17 | SOT-23-5 | 3 | PG, wake, alarm, shutdown, gate |
| Hex Schmitt inverter | SN74LVC14A | TSSOP-14 | 1 | Industrial header inputs |

## 10.5 Power Section (14 components, bottom side)

| Component | Part Number | Package | Src | Notes |
| --- | --- | --- | --- | --- |
| MPPT charger | CN3722 | SOP-16 | LCSC | Solar→LiFePO4 |
| Boost 5V | TPS61022DRLR | SOT-23-6 | JLC | Batt→5V@4A |
| PoE PD | SI3402-B | QFN/SOP | LCSC | 802.3at isol. |
| USB-C PD | HUSB238 | QFN-16 | JLC | 12V negotiate |
| OR-ing | LTC4357 | SOT-23-5 | JLC | Auto-switch |
| LDO (RP) | MIC5219-3.3YM5 | SOT-23-5 | JLC | 3V3 always-on |
| LDO (3506) | AP2112K-3.3 | SOT-23-5 | JLC | 3V3 always-on |
| PMOS switch | Si2301 | SOT-23 | JLC | Brain power |
| Ferrite ×3 | BLM18AG102SN1D | 0603 | JLC | Domain entry |
| USB ESD ×4 | USBLC6-2SC6 | SOT-23-6 | JLC | USB ports |
| HDMI ESD | PRTR5V0U2X | SOT-363 | JLC | HDMI port |
| Boost inductor | SWPA4012S 1µH | 4012 | LCSC | TPS61022 |
| Battery conn | JST PH 2-pin | THT | LCSC | LiFePO4 |
| Solar conn | JST VH 2-pin | THT | LCSC | Panel input |

# 11. PCB Routing Guidelines

## 11.1 Routing Priority Order

- DDR CH0+CH1 (L1/L3). Length-match ±25mil within byte lanes. ROUTE FIRST.

- NVMe PCIe x2 diff (L3). 85Ω, ±5mil matched, no 90° bends.

- MIPI CSI diff (L1). 100Ω, ≤30mm. Match pairs.

- HDMI TMDS diff (L3). 100Ω, ≤60mm.

- USB 2.0 D+/D− (L1). 90Ω, ≤25mm, no 90° bends.

- RGMII Ethernet (L1). 50Ω, ≤40mm.

- SDIO WiFi 5 (L1). 50Ω, ≤30mm, ±5mm length match to CLK.

- I2S Audio (L1). Match BCLK/LRCK/DATA lengths.

- CYW43439 SPI + LoRa SPI (L6). 50Ω, ≤20mm.

- CAN/RS485 (L6). Not impedance-controlled. Keep short.

- Power planes (L4). Copper fills for 5V, 3V3, 1V8, VBAT.

- Everything else (UARTs, I2C, GPIO, SPI bridges, cellular USB)

## 11.2 Critical Layout Rules

- Via-in-pad (POFV) on RK3576 BGA — free on JLCPCB 6-layer orders

- 100nF cap within 0.5mm of EVERY RK3576 VDD ball (verify in 3D view)

- WiFi antenna keepout: no copper on ANY of 6 layers within 3mm

- CAN/RS485 isolation barrier: no copper crossing between isolated and non-isolated

- PoE isolation: maintain creepage/clearance per IEC 62368-1

- MPPT switching node: short, direct, no ground copper under inductor

- RP2350 domain: GND island with single connection back to main GND

- Ferrite beads placed at physical domain boundaries on 5V trace

- Schmitt trigger ICs placed directly at domain boundary crossings

# 12. Reference Documents

| Document | Source | Use For |
| --- | --- | --- |
| RK3576 Hardware Design Guide V1.1 | Scribd: search “Hardware Design 3576 en” | DDR, PMIC, USB, PCIe — PRIMARY |
| ArmSoM Sige5 schematic | docs.armsom.org | RK3576+RK806 exact rail voltages |
| Pico 2W schematic | datasheets.raspberrypi.com | CYW43439 SPI wiring — copy exactly |
| RP2350 HW design guide | datasheets.raspberrypi.com | RP2350A minimal circuit, SMPS, crystal |
| RK3506G2 datasheet | opensource.rock-chips.com | Full peripheral map, pin mux, FLEXBUS |
| Firefly Core3506BY BSP | en.t-firefly.com | RK3506J AMP config, CAN/RS485 drivers |
| Quectel EC25 HW design guide | quectel.com | 4G LTE integration, USB, antenna |
| ES8316 datasheet | LCSC / Everest Semiconductor | Audio codec I2S wiring, analog design |
| SX1262 datasheet | semtech.com | LoRa RF matching, SPI commands |
| SN74LVC1G17 / 2G17 datasheet | ti.com | Schmitt trigger application circuits |
| SN74LVC14A datasheet | ti.com | Hex Schmitt trigger specs |
| CN3722 datasheet | LCSC | MPPT circuit, charge current programming |
| TPS61022 datasheet | ti.com | Boost converter, inductor selection |
| SI3402-B datasheet | Silicon Labs | PoE PD circuit, isolated DC-DC |
| JLCPCB impedance calculator | jlcpcb.com/pcb-impedance-calculator | Trace widths for JLC06161H-3313 |
| JLCPCB stackup files | github.com/gsuberland/jlcpcb_autogenerated_stackups | Pre-built KiCad stackup import |
| PCBWay RK3576 SBC V1.0 | pcbway.com (CERN-OHL-P license) | KiCad reference design, symbol/footprint |

# 13. Ordering Information

| JLCPCB Setting | Value |
| --- | --- |
| Layers | 6 |
| Stackup | JLC06161H-3313 (specify in order notes) |
| Surface finish | ENIG |
| Impedance control | YES |
| Via-in-pad (POFV) | YES (free on 6+ layers) |
| X-Ray inspection | YES (RK3576, LPDDR4X, eMMC BGAs) |
| Material | FR-4 TG155 |
| Assembly | Standard PCBA — BOTH SIDES |
| Estimated cost (5 pcs) | USD $350–500 assembled |
| Order notes | Impedance: 50Ω SE, 85/90/100Ω diff. Stackup: JLC06161H-3313. |

# 14. Revision History

| Rev | Date | Changes |
| --- | --- | --- |
| 1.0 | Jan 2026 | Initial RK3576+RP2350 dual-processor spec (no RK3506J) |
| 2.0 | Feb 2026 | Added RK3506J automation domain, cellular, solar, PoE |
| 3.0 | Mar 2026 | Added Schmitt triggers, ferrite filtering, expanded headers, cameras, audio, all USB-C, full datasheet format |

Solarpunk Pi v3.0 Three brains, three power sources, five networks, thirty automation I/O pins, Schmitt-triggered signal conditioning, ferrite-filtered power rails, ESD-protected connectors — in a board the size of a credit card. Solar-powered edge computing with real-time industrial automation.